# Project 2 Proposal: Obstacle Avoidance Grasping for Robot Arm Based on Deep Reinforcement Learning

| DU XUAN A0329059W | WANG ZHIMING A0331406R | ZUO RONGXIN A0331562L |
| --- | --- | --- |

## 1. Motivation (WHY?)

In modern industrial automation and domestic service robotics, robots must operate in non-structured environments where static or semi-static obstacles are present. Traditional fixed-path programming is insufficient for dynamic tasks where the target or obstacle positions vary. Developing an agent capable of learning "eye-hand" coordination allows for more flexible deployments, such as picking items from cluttered shelves or performing precision assembly in cramped spaces. Deep Reinforcement Learning (DRL) offers a way to learn these complex mappings directly from high-dimensional sensor data to joint commands.

## 2. Conventional Algorithms

- Inverse Kinematics (IK): Traditionally, the goal position is converted to joint angles using Jacobian-based or optimization methods. However, IK often struggles with singularity points and does not inherently account for obstacle avoidance.

- Sampling-based Planners (RRT/PRM): These are state-of-the-art for obstacle avoidance but require a complete global map and can be computationally expensive to re-run if the environment changes slightly.

- ML Advantage: RL allows for online planning and reactive control. Once trained, the policy can infer the next move in milliseconds, enabling the robot to start moving immediately without waiting for a full pre-computed global path.

## 3. Problem Statement (WHAT?)

The environment consists of a 3-DOF robotic arm in a 3D workspace with randomly placed static obstacles and a target object.

- Robot Capability: The agent controls the continuous joint velocities or position increments of the arm.

- Sensing: The robot receives its current joint states (angles and velocities) and the 3D coordinates of the target object. Obstacle information is obtained via a simulated depth sensor with a limited sensing range, meaning the agent can only detect obstacles within a defined radius around the end-effector. This reflects the real-world constraint that physical robots rely on onboard sensors and do not have access to a complete global environment map.

- Goal: Reach the end-effector to the target coordinates within a specific tolerance, while ensuring no part of the arm collides with obstacles. The entire environment and robot dynamics will be implemented using the MuJoCo physics engine, which provides high-fidelity continuous simulation and accurate contact dynamics suitable for robotic control tasks.

## 4. RL Cast (HOW?)

To solve this, we cast the problem into the framework:

- State Space (S):

  - Proprioception: Current joint angles and joint velocities.

  - Task Context: 3D relative vector from the end-effector to the target and the Euclidean distance.

  - Obstacle Data: To respect the robotic inspiration of local sensing, the agent receives distances from a suite of virtual range sensors (simulated ray-casting in MuJoCo) mounted on the arm links, which detect obstacles within a 0.5m radius.

- Action Space (A):

  - Actions represent the target angular velocity for each of the 3 joints, scaled to [-1, 1] to match the motor's physical limits.

- Reward Structure (R):

  - Distance Reward: A positive reward proportional to the reduction in Euclidean distance between the end-effector and the target.

  - Action Smoothing Penalty: Negative reward on timesteps to minimize mechanical jitter and stabilize learning.

  - Collision Penalty: A large negative constant if any link hits an obstacle.

  - Success Reward: A large positive constant upon reaching the goal.

  - Efficiency: A small penalty for each timestep to encourage the fastest path.

- Neural Network Structure:

  - We use an Actor-Critic architecture built on MLPs. The proprioceptive data and task context are concatenated and passed through a shared MLP backbone, while obstacle data is processed by a separate MLP branch. The outputs of both branches are then concatenated and fed into the final actor head and critic head respectively. As an optional extension, a CNN backbone may replace the obstacle MLP branch if depth map inputs are incorporated.

## 5. RL Algorithms and Envisioned Results

We will utilize Soft Actor-Critic (SAC) as the primary reinforcement learning algorithm, implemented within the MuJoCo physics engine. SAC is uniquely suited for this 3-DOF robotic arm task because its maximum entropy objective encourages extensive exploration in the continuous joint-velocity space, allowing the agent to discover smoother and more diverse paths around obstacles compared to on-policy methods.

To evaluate the effectiveness of the learned policy, we will conduct a three-way comparison between our SAC-based reactive controller, a traditional RRT* planner, and a standard Inverse Kinematics (IK) baseline. The performance will be measured across the following metrics:

- Success Rate: Percentage of episodes where the end-effector reaches the target within tolerance without any part of the arm colliding with obstacles.

- Planning vs. Execution Latency: We will measure the time from goal assignment to the first joint movement. We expect SAC to show near-zero planning latency (reactive control), whereas RRT* will require significant pre-computation for each new obstacle layout.

- Collision Robustness: We will evaluate the collision rate in cluttered environments. While IK is expected to fail or collide, and RRT* may struggle with real-time adaptations, SAC is expected to exhibit superior reactive obstacle avoidance.

- Motion Smoothness: Measured by joint velocity variance to ensure the learned control is physically stable and reduces mechanical wear.

These experiments will be conducted under varying obstacle densities (1, 3, 5 obstacles) and target configurations. We expect that while RRT* provides optimal static paths, our SAC agent will offer superior reactive adaptability and immediate execution speed, proving the advantages of DRL for online robotic control.